I did have a question for you Chris, how are you currently calculating the coefficient of variation variables we have? I know we briefly spoke about this before, but I don't recall an exact formula being discussed. Either way, I look forward to feedback from both of you and getting the automated autoresuscitation pipeline completed!

For irregularity score it is the absolute value of the difference between the current breath and the previous breath, divided by the previous breath

|TT[N] - TT[N-1]|/TT[N-1]

Definition of when to switch gas

- At end of first apnea (30 seconds without breath)

We have been setting this at 3 seconds, but can certainly explore changing this

This was based on methods described by Kevin Cummings – I realized this was different from ours, but I’m not sure what the benefits/ detriments are. He defines an apnea as 30 seconds without a breath. So if we are to switch gas at onset of apnea, it may be prudent to change this setting? Although I’m sure Russell has a reasoning behind setting it at 3 seconds rather than 30.

Definition of recovery criteria

- Recovery of HR and respiratory rate to 63% of baseline average

This is what we are currently using

Yes, I kept this the same. Kevin uses 90% recovery. We can discuss changing this?

Calculated and recorded outcomes per bout (comparisons between bouts)

- Gasp latency

  - Time from initial onset of primary apnea to initial gasp

- Gasp frequency

  - Number of gasps until recovery of respiratory rate to 63%

How do we want to define a gasp vs a non-gasp breath during the recovery period

I think this should be defined as an inspiratory event that is not followed by another inspiratory event at a rate 50% or below average respiratory rate established in baseline. 50% is arbitrary, but a start. Perhaps a machine learning technique could be applied to this?

- Respiratory recovery time

  - Total time from initial onset of primary apnea to recovery of 63% of baseline respiratory rate

Because breathing and heart rate values are being defined per breath/beat, do we want to include a requirement for a sustained recovery or use the first instance on the instantaneous values reaching the recovery threshold?

This is a great point. I think it should be sustained recovery as it may be possible to reach “recovery” parameters instantaneously without fully recovering. But I will refer to Russell on this.

- HR recovery time

  - Total time from initial onset of primary apnea to recovery of 63% of baseline HR

- Exposure time to challenge gas

  - Time from onset of challenge gas to beginning of apnea

Calculated and recorded outcomes per recording period outside of bouts (comparisons between baseline recording, recovered breathing after first hypoxic episode, baseline post-CNO, and recovered breathing after first hypoxic episode post-CNO)

- Respiratory rate

- Metabolic rate (we aren’t capturing O2 or CO2 concentrations, so I don’t think we can calculate this)

- Ventilation

- Respiratory equivalent (is this the same as respiratory exchange ratio?, if so, comment for metabolic rate will also impact this)

Yes, it is.

- VCO2 (we aren’t capturing O2 or CO2 concentrations, so I don’t think we can calculate this)

- Tidal volume

- VTG

- Low frequency HR variability (what are the frequency cut-offs that people are using for high freq vs low freq?)

Low frequency = 0.5-1.5 Hz and high frequency = 1.5-6 Hz (Just et. al., 2000)

- High frequency HR variability

- LF/HF ratio

- Coefficient of variation of respiratory rate (are they calculating this with the formula I describe above, or a different formula?)

This is unclear. I don’t see a formula reported so far. But I can email the authors to find out their definition.